Vulnerability Report: D-Link DAP-2310 atp /xgi /cfg_valid.xgi DoS via unsafe string handling

Affected Software

Vendor: D-Link

Product: DAP-2310 (wireless access point)

Tested Version: dap2310_v1.00_o772.bin

Affected Component: atp (CGI handler), process: xgi

Affected Endpoint: /cfg_valid.xgi

Architecture: MIPS (24Kc)

Disassembly Base Address: 0x400000

Vulnerability Overview

This report consolidates two crash reports affecting the same CGI endpoint (/cfg_valid.xgi) in the D-Link DAP-2310 firmware. A crafted HTTP request can cause the xgi CGI handler to terminate (SIGSEGV), resulting in denial of service for the web management interface. Two distinct crash manifestations were observed: (1) memory corruption consistent with a stack-based buffer overflow (attacker-controlled BadVA 0x7a79777a) and (2) a NULL pointer dereference in rpsubname() when processing an input string lacking a '.' character.

Vulnerability Details

Vulnerability Type: Remote Denial of Service (xgi crash); memory corruption variant may be exploitable depending on mitigations

CWE: CWE-121 (Stack-based Buffer Overflow) and CWE-476 (NULL Pointer Dereference); underlying input validation weakness (CWE-20)

Attack Vector: Remote (HTTP management interface)

Authentication: Likely required (both PoCs start with POST /login.php; verify default/blank-password behavior on target)

Impact: DoS of web management interface (xgi process crash). Potential code execution risk for the memory-corruption variant requires further analysis.

Triggering Request Pattern: Both reports indicate a request sequence (delimiter \x1a\x1a\x1a\x1a) consisting of a login request followed by a crafted GET to /cfg_valid.xgi with an unusually long query string and/or repeated parameters (notably repeated exeshell parameters with at least one very long value).

Crash Signature A (Memory Corruption): BadVA=0x7a79777a; fault in libc.so.0; call stack repeatedly traverses atp::redirect_htm (0x403830–0x40390c) and atp::rpsubname (0x403a18–0x403a68) before crashing.

This signature shows attacker-controlled bytes in the faulting address (0x7a79777a, corresponding to ASCII-like pattern bytes), which is consistent with a pointer or saved state being overwritten. The report attributes the likely cause to unsafe C string operations (e.g., strcpy/strcat/sprintf) on fixed-size buffers without length checks when handling the long query string.

Crash Signature B (NULL Pointer Dereference): BadVA=0x00000000; RA=0x00403a58 (atp) returning from libc strcpy; invalid write to NULL.

In this variant, rpsubname() calls strrchr(param_1, '.') and immediately uses the returned pointer as the destination for strcpy(). If the input string does not contain '.', strrchr() returns NULL and strcpy(NULL, ".") triggers a NULL-pointer write.

Likely Root Cause

The /cfg_valid.xgi handler processes attacker-controlled query strings using helper routines that assume well-formed inputs and use unsafe string primitives. In rpsubname(), the result of strrchr() is not validated before being passed to strcpy(), producing a deterministic NULL pointer dereference when the expected delimiter '.' is absent.

Separately, handling of repeated parameters and very long values (e.g., repeated exeshell parameters) appears to involve unbounded concatenation/formatting into fixed-size buffers inside redirect_htm()/rpsubname() call paths, leading to memory corruption when inputs exceed expected lengths. The observed attacker-derived BadVA (0x7a79777a) supports an overwrite of a pointer used by libc.

Mitigation / Remediation

• Validate all CGI inputs (length, character set, expected delimiters) before processing.
• Replace strcpy/strcat/sprintf with bounded alternatives (snprintf/strlcpy/strlcat) and enforce explicit maximum lengths.
• In rpsubname(), check strrchr() return value before use and handle missing '.' safely.
• Add request-size limits and reject repeated parameters or excessive query lengths.
• Enable and verify hardening mitigations (stack canaries, NX, PIE/ASLR, RELRO, FORTIFY_SOURCE).