Vulnerability Report: DIR-300 V1.03_7c (udhcpd / inet_aton)

Affected Software

Vendor: D-Link

Product: DIR-300

Tested Version: V1.03_7c

Affected Component: udhcpd (DHCP server); triggered via bsc_lan.xgi / web management

Vulnerability Overview

A remote attacker can trigger a crash (SIGSEGV) in the DHCP server component by driving the firmware into a state where a NULL pointer is passed as an IP/hostname string to an address-parsing routine. The vulnerable code passes a NULL pointer to inet_aton(), resulting in an invalid read at address 0x00000000 inside libc and termination of the udhcpd process.

Vulnerability Details

Vulnerability Type: Remote crash / Denial of Service via NULL pointer dereference

CWE: CWE-476 (NULL Pointer Dereference)

Attack Vector: Remote (HTTP request sequence culminating in bsc_lan.xgi actions that restart/reconfigure DHCP)

Authentication: required

Impact: Denial of Service (udhcpd crash; DHCP service disruption for LAN clients)

Observed Affected Handlers / Functions: udhcpd: FUN_00402f70() → inet_aton()

Proof of Concept (PoC): The PoC is an HTTP request sequence of multiple requests separated by the 4-byte delimiter: `\x1A\x1A\x1A\x1A`. The sequence performs an authenticated session (login.php), sets temporary nodes (set_temp_nodes.php), updates LAN/DHCP settings (bsc_lan.php), then triggers actions via bsc_lan.xgi (`exeshell=submit COMMIT` and `exeshell=submit DHCPD`) to reach the crashing state.

Likely Root Cause

udhcpd forwards an attacker-influenced pointer (derived from configuration/state manipulated through the web interface) into FUN_00402f70(), which directly passes its first argument to inet_aton() without validating that it is non-NULL. When the field is missing/empty or becomes NULL due to an unexpected state transition, inet_aton() dereferences NULL inside libc and the process terminates with SIGSEGV.

The defect is consistent with insufficient NULL checks and input/state validation at the boundary where configuration values are read or derived before being passed to libc address-parsing routines.