Vulnerability Report – D-Link DIR-300 (xmldb)

Affected Software

Vendor: D-Link

Product: DIR-300

Tested Version(s): v1.03_7c

Affected Component: xmldb (user-space daemon handling configuration/database operations)

Architecture: MIPSEB (as tested)

Vulnerability Overview

This report describes a memory corruption vulnerability in the DIR-300 firmware. A crafted sequence of HTTP requests can cause the xmldb process to crash with a segmentation fault. The crash occurs while xmldb builds a response string using unsafe C string operations (e.g., strcat/strcpy), resulting in an invalid write.

Impact

- Remote Denial of Service (DoS): xmldb terminates due to SIGSEGV.

- Potential Memory Corruption: the fault is triggered during string concatenation; further exploitability (e.g., code execution) has not been confirmed.

Vulnerability Details

Vulnerability Type: Buffer overflow / memory corruption via unsafe string concatenation

CWE: CWE-119 (Improper Restriction of Operations within the Bounds of a Memory Buffer)

Crash Type: Segmentation Fault (SIGSEGV) – invalid write

Vulnerable Binary: xmldb

Key Function(s): FUN_0040b7e4 (base 0x400000), libc strcat/strcpy/strncpy usage

Likely Root Cause

The PoC drives the web interface to create/update temporary nodes and then commits configuration changes. A long attacker-controlled field (observed in parameter d_2_3) propagates into xmldb and reaches FUN_0040b7e4, where a stack buffer (sp+0x218) is populated and later concatenated using strcat. With sufficiently long input, the destination buffer is exceeded, leading to an invalid write and crash.

POST /login.php HTTP/1.1\r\nHost: 192.168.0.1\r\n...\r\n\r\nACTION_POST=LOGIN&LOGIN_USER=admin&LOGIN_PASSWD=&login=+Log+In+

\x1A\x1A\x1A\x1A

POST /set_temp_nodes.php HTTP/1.1\r\nHost: 192.168.0.1\r\n...\r\n\r\nTEMP_NODES=/runtime/post/session_1&data=4&start=1&...

...&d_2_3=<very long attacker-controlled value>&...&end=25

\x1A\x1A\x1A\x1A

POST /bsc_lan.php HTTP/1.1\r\nHost: 192.168.0.1\r\n...\r\n\r\nACTION_POST=SOMETHING&lease_seconds=604800&...&leasetime=10080

At crash time, the kernel log indicates an invalid write at address 0x7fa2a000 and a return address within xmldb (ra=0x0040bb5c). The call stack shows strcat as the faulting primitive, reached from xmldb code at offsets 0x0000bb44/0x0000bb5c (base 0x400000).

Proof of Concept (PoC): The PoCs are HTTP request sequences of multiple requests separated by the 4-byte delimiter: `\x1A\x1A\x1A\x1A`. The “PoC” directory contains the PoCs to reeproduce the bug