Vulnerability Report: TRENDnet TV-IP121WN view.cgi SIGSEGV via malformed oversized HTTP headers

Affected Software

Vendor: TRENDnet

Product: TV-IP121WN

Tested Version: FW_TV-IP121WN_1.2.2.zip

Affected Component: view.cgi (web management CGI)

Affected Endpoint: /admin/view.cgi?proLile=2&language=en

Attack Vector: Remote (HTTP)

Authentication: Required (HTTP Basic Authentication observed in PoC)

Architecture: MIPS 24Kc

Disassembly Base Address: 0x400000

Vulnerability Overview

An authenticated remote attacker can crash the TV-IP121WN administrative CGI handler view.cgi by sending a crafted HTTP request containing malformed and oversized header content. The request results in a SIGSEGV due to an invalid read from address 0x00000000, causing denial of service of the device web management functionality.

Vulnerability Details

Vulnerability Type: Remote Denial of Service (crash); suspected memory-safety issue in request parsing/CGI env handling

CWE: CWE-476 (NULL Pointer Dereference) / related: CWE-20 (Improper Input Validation)

Impact: DoS of web management functionality (view.cgi crash; may affect request servicing through boa)

Crash Type: SIGSEGV (invalid read at NULL)

BadVA: 0x00000000

EPC: 0x76eed550 in libuClibc-0.9.30.so

RA (in view.cgi): 0x00400e90 (view.cgi base 0x400000)

Relevant Disassembly Context: view.cgi::main around 0x00400d40–0x00400e90; RA corresponds to post-atoi path

Proof of Concept (PoC): Send a single GET request to /admin/view.cgi containing malformed/oversized headers. The original PoC includes long header values with embedded non-printable bytes (e.g., '\x00', '\x01') and large repeating patterns (e.g., 'zyw...') that trigger the crash.

Example (abridged):
GET /admin/view.cgi?proLile=2&language=en HTTP/1.1\r\nHost: 127.0.0.1\r\nAuthorization: Basic YWRtaW46YWRtaW4=\r\n<malformed/oversized headers ...>\r\n\r\n

Technical notes: The parent web server (boa) trace indicates CGI environment construction (init_cgi/complete_env/add_cgi_env/create_common_env) immediately prior to view.cgi execution. The self trace places view.cgi execution in main, with the recorded return address at 0x00400e90 and the faulting instruction in libuClibc.

Likely Root Cause

The crash appears to be driven by improper handling of malformed header data during CGI environment construction and/or view.cgi argument parsing. Oversized header names/values and embedded non-printable bytes may cause internal parsers to treat pointers as missing/uninitialized, leading to a NULL pointer being passed into a libc routine. The kernel log reports an invalid read from 0x00000000 and an EPC inside libuClibc, consistent with a NULL pointer used by a string/memory routine.

Because the parent trace shows extensive environment-variable building in boa, one plausible trigger is a failure in CGI env generation (e.g., incomplete/malformed key/value pairs) that propagates an unexpected NULL into view.cgi or into a libc helper used by view.cgi. Further analysis of the exact failing libc symbol at EPC 0x76eed550 would refine the specific root cause.

Mitigation / Remediation

Vendor remediation:
• Enforce strict maximum lengths for HTTP header names/values and total header size.
• Reject or sanitize non-printable bytes in headers before CGI environment creation.
• Add NULL checks and error handling around all header/CGI parameter extraction before calling libc routines.
• Prefer bounded string APIs and centralized limits when constructing CGI environment variables.

Operational mitigations:
• Restrict access to the administrative interface to trusted networks.
• Disable remote administration exposure where possible.