Vulnerability Report: TRENDnet TV-IP651WI alphapd multipart/form-data parsing Out-of-bounds Read

Affected Software

Vendor: TRENDnet

Product: TV-IP651WI (Wireless Day/Night PTZ Network Camera)

Tested Version: FW_TV-IP651WI_V1_1.07.01.zip

Affected Component: alphapd (embedded web server)

Affected Endpoints (observed): /setSystemControl, /setFirmwareUpgrade

Vulnerability Type: Out-of-bounds read / invalid pointer dereference during multipart parsing

CWE: CWE-125 (Out-of-bounds Read)

Authentication: Required (Digest authentication shown in PoC)

Disassembly Base Address: 0x400000

Vulnerability Overview

A segmentation fault (SIGSEGV) was identified in alphapd as shipped in FW_TV-IP651WI_V1_1.07.01.zip for the TRENDnet TV-IP651WI network camera. A remote authenticated attacker can trigger a crash by sending a sequence of HTTP POST requests culminating in a crafted multipart/form-data upload to the firmware upgrade handler. The crash occurs during request parsing and results in termination of the alphapd web server process, causing a denial of service of the web management interface.

Vulnerability Details

Attack Vector: Remote (HTTP)

Impact: Authenticated Denial of Service (alphapd crash)

CVSS v3.1 (Estimated): 4.9 (CVSS:3.1/AV:N/AC:L/PR:H/UI:N/S:U/C:N/I:N/A:H)

Crash Type: SIGSEGV (invalid read)

BadVA (invalid read address): 0x008e562c

EPC: 0x772c9114 (libuClibc-0.9.28.so)

RA: 0x772c90b0 (libuClibc-0.9.28.so)

High-level Call Chain: websUrlProcessRequest → websCgibinProcessor → findStrInFile → memmem → libuClibc

Proof of Concept (PoC): The crash is triggered by sending two HTTP requests separated by the delimiter \x1a\x1a\x1a\x1a. First, POST /setSystemControl (application/x-www-form-urlencoded). Second, POST /setFirmwareUpgrade with Content-Type multipart/form-data and a crafted boundary value. The multipart body includes parameters such as ReplySuccessPage, ReplyErrorPage, ForceBootCodeUpgrade, a firmware file part (DownloadFile), and DownloadCSRV=Upgrade, with the boundary string manipulated to be malformed/overlong.

Likely Root Cause

alphapd's CGI/multipart request parsing is orchestrated by websCgibinProcessor. During firmware upgrade handling, helper routines (findStrInFile → memmem) scan uploaded request data for multipart boundary markers and header delimiters. Under malformed or extreme boundary conditions, the scanning logic can operate with invalid buffer pointer/length combinations or incorrect chunk-boundary arithmetic, resulting in an out-of-bounds read inside libuClibc search/comparison routines (fault at EPC 0x772c9114).

The observed call chain into memmem and the invalid read address (BadVA 0x008e562c) are consistent with a parser bug where the search cursor advances beyond the valid buffer range when attempting to match attacker-supplied boundary text.

Mitigation / Remediation

Vendor remediation:
• Harden multipart/form-data parsing: validate boundary length/charset and reject excessively long or malformed boundaries.
• Validate all file/buffer pointers and lengths before passing them to memmem/memchr/memcmp.
• Add defensive checks when scanning across chunk boundaries to prevent pointer arithmetic from producing invalid addresses.
• Fuzz test multipart parsing and the firmware upgrade handler to identify additional edge cases.

Operational mitigations:
• Restrict access to the management interface to trusted networks and enforce strong admin credentials.
• Disable remote administration exposure where possible.