Vulnerability Report: Linksys WRT320N httpd /apply.cgi multiple DoS conditions in CGI parsing and MAC handling

Affected Software

Vendor: Linksys

Product: WRT320N wireless router

Tested Version: FW_WRT320N_1.0.05.002_20110331.bin

Affected Component: httpd (embedded web server)

Affected Endpoint(s): /apply.cgi (multiple handlers: WanMAC, Wireless_MAC, SingleForward)

Architecture: MIPS (24Kc)

Disassembly Base Address: 0x00400000

Vulnerability Overview

This report consolidates five related crash reports impacting the Linksys WRT320N embedded web server (httpd). Each crash is triggered via crafted HTTP POST requests to /apply.cgi and results in termination of httpd (denial of service of the management interface). The observed failures fall into two broad categories: (1) memory corruption in the WanMAC (MAC cloning) flow when MAC fields are malformed or overlong, including stack-based overflow and attacker-influenced pointer dereferences; and (2) NULL pointer dereferences in other /apply.cgi handlers (Wireless MAC filter and Single Forward) due to missing/uninitialized CGI parameters.

Vulnerability Details

Vulnerability Type: Remote Denial of Service; memory-corruption variant may be exploitable depending on mitigations

CWE: CWE-121 (Stack-based Buffer Overflow), CWE-822 (Untrusted Pointer Dereference), CWE-476 (NULL Pointer Dereference)

Attack Vector: Remote (HTTP POST to /apply.cgi)

Authentication: Required (PoCs use HTTP Basic Authentication; e.g., admin:admin)

Impact: DoS (httpd crash). Potential RCE risk for stack overflow variant requires additional analysis.

Primary code paths implicated in the WanMAC crashes (from traces): validate_cgi → get_cgi → get_merge_mac → valid_hwaddr → legal_hwaddr.

Issue Cluster A (WanMAC / MAC clone handling): submit_button=WanMAC; parameters def_hwaddr_*

Three reports describe crashes while processing the WanMAC (MAC cloning) handler. These are triggered by malformed or oversized MAC component fields (def_hwaddr_1/2/4). Crash artifacts show either (a) stack corruption with EPC/RA overwritten by attacker-controlled bytes (EPC=RA=0x797a7779; BadVA=0x797a7778), or (b) attacker-influenced pointer dereferences where BadVA resembles ASCII fragments derived from request data (e.g., BadVA=0x303A3048, "0:0H"). A fourth related report describes a general SIGSEGV in the same WanMAC/MAC-validation path with BadVA values consistent with user-controlled bytes.

PoC notes (WanMAC): POST /apply.cgi with submit_button=WanMAC and mac_clone_enable=1. Trigger examples include: • def_hwaddr_1 set to an excessively long string (overflow), • def_hwaddr_2 containing an embedded NUL and non-hex characters, • def_hwaddr_4 set to a very long attacker-controlled string.

Issue Cluster B (Wireless_MAC filter NULL dereference): submit_button=Wireless_MAC; parameters m0..mN

One report describes a NULL pointer dereference during Wireless MAC filter processing, where valid_hwaddr() is invoked with a NULL MAC string pointer. In the captured state, a libc memcmp-like routine is called with a0=NULL (BadVA=0x00000000), and execution returns to httpd at 0x00428bd0 (base 0x00400000). The provided PoC indicates at least one MAC entry field (e.g., m3=) is left empty/missing, allowing the CGI lookup to return NULL which is then dereferenced.

Issue Cluster C (SingleForward NULL dereference): submit_button=SingleForward; protocol parsing

One report describes a NULL pointer dereference in the Single Forward (port forwarding) handler. The call chain includes find_proto and validate_forward_single, and the fault occurs at EPC=0x00448410 with BadVA=0x00000000 (RA=0x004483b8). This indicates an uninitialized or missing pointer derived from CGI parameters is used without validation.

Likely Root Cause

Across the five reports, the common underlying weakness is unsafe CGI parameter handling in /apply.cgi: parameters are assumed to be present, well-formed, and within expected size limits. In the WanMAC handler, attacker-controlled strings appear to be copied/merged into fixed-size buffers and/or used to populate pointer-bearing structures without robust bounds checking or sanitization, producing memory corruption and attacker-influenced pointer dereferences. In the Wireless_MAC and SingleForward handlers, missing/empty parameters propagate NULL pointers into string comparison/validation routines (memcmp/strcmp/other), leading to deterministic NULL pointer dereferences.

These behaviors are consistent with insufficient input validation (CWE-20) combined with unsafe string handling (CWE-121/CWE-822) and missing NULL checks (CWE-476).

Mitigation / Remediation

Vendor remediation:
• Enforce strict input validation for all /apply.cgi handlers (required fields, allowed character sets, exact lengths).
• Apply hard bounds for all MAC-address-related components; reject embedded NULs and non-hex characters.
• Replace unbounded copies/concatenations with bounded operations (snprintf/strlcpy/strlcat) and propagate errors.
• Add NULL checks for all pointers returned by CGI parameter retrieval before use in libc routines.
• Consider enabling/strengthening exploit mitigations (stack canaries, NX, ASLR/PIE if supported, FORTIFY_SOURCE).

Operational mitigations:
• Restrict management interface exposure (LAN-only/VPN/firewall) and enforce strong credentials.
• Rate-limit inbound requests and reject abnormally large request bodies at the edge.